Please read the scenario below and respond to questions in your submission.

Black Ice Manufacturing makes wooden cabinets for high-end audio speakers that are finished in a high-gloss black paint. Because of the premium charged for such speakers, customers expect the finish on such speakers to be near flawless. Over the years, Black Ice has perfected the finish painting process and has established itself as a reliable supplier of high-gloss cabinets. Recently some customers have been complaining about the paint finish. Mary Snow, the manufacturing manager of Black Ice, has been tasked with investigating the problem.

Mary first reviews the quality control procedures and learns that each speaker is inspected to look for “visible” blemishes (defined as blemishes of size larger than 0.5 mm). Speakers with one or more visible blemishes were rejected as being defective. Based on her conversations with the quality department, Mary is assured that there has not been any change in process settings or quality control standards. Mary is concerned that the standard for “visible” blemishes may be inappropriate and the reason for the problem. In particular, she suspects that smaller blemishes may not be individually visible, but a large number of smaller blemishes may cause customers to feel that the finish is of poor quality. Mary has asked the quality control department to investigate the problem further. She asks for random samples to be collected every hour during two eight-hour shifts, and for every sampled cabinet be inspected under magnification and any blemish larger than 0.1 mm be noted. The quality control department collected 10 samples every hour and recorded the number of blemishes larger than 0.1 mm on each sampled cabinet. The data from the 160 cabinets (10 samples/hour X 16 hours) is given in the file attached below. For each sample the file shows the number of “visible” blemishes (larger than 0.5mm) and the number of blemishes between 0.1mm and 0.5mm observed under magnification.

Based on this information and the data given, please answer Question 1 through 4 below.

Question 1: Mary Snow has asked you to look at the collected data and determine which data to use and what type of chart would be appropriate in this case. Briefly justify your answer. (3 points)

The company is receiving more complaints on the quality of the cabinets despite the number of defects reported under the existing quality control standards remaining unchanged. Mary should use the data of blemishes larger than 0.1mm but smaller than 0.5mm to test her suspicion that the customer complaints are as a result of an excessively large number of smaller blemishes. Since Mary is concerned about the frequency of less visible blemishes in the 0.1mm to 0.5 mm, she should use the x-control chart to determine whether a significant number of the sampled cabinets have minor blemishes that exceed the acceptable level.

Question 2: If a C-chart is used to analyze the blemishes between 0.1 and 0.5mm, calculate the center line, and the lower and upper control limits. (You do not have to draw the control chart) (3 points)

Center line = Mean of sample means

Center line = 8.875

Upper Control Limit = Grand mean + A2 R

A2 for n=10: 0.308

R = 10.938

UCL = 8.875 + 0.308*10.938

UCL = 12.244

LCL = 8.875 – 0.308*10.983

LCL = 5.506

Question 3: Interpret the data by comparing the sample data you used with your calculated control limits (2 points)

Of the 160 sampled cabinets, 35 had more than 12 blemishes, which is the upper control limit. The observations falling above the UCL represent 21.88% of the total sample. Thirty cabinets (18.75%) had fewer than 5 blemishes, which is the lower control limit. While few blemishes are desirable, the high frequency of observations outside both limits indicates high variability in the production process with regard to smaller blemish control.

Question 4: Does the data and your analysis above, support Mary Snow’s belief that customer complaints are the result of many smaller blemishes. (2 points)

The data shows that the process is out of control with respect to the prevention of defects in the 0.1mm to 0.5 mm category. On average customers are receiving cabinets with approximately 9 small blemishes with an estimated 22% of customers receiving cabinets with more than 12 blemishes. Despite the wide control limit, with allowed variation of 3.37 points above and below the center line, 40.63% of the sampled cabinets fell outside the control limits. The data supports Mary’s belief that customer complaints are the result of many smaller blemishes. The company should create quality control standards for the acceptable number of smaller blemishes and design quality control processes to identify cabinets that exceed the target.